# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title : Supply of Equipment for Metal Hardening Process			p 1 / 5

Publication reference : IPAECOHARD01

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Lot 1 – Furnaces with Noble Gases and Other Equipment

| 1. Item Number | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | FURNACE WITH HELIUM GAS ATMOSPHERE, Quantity 1 |  |  |  |  |
| 1.1. | General specifications |  |  |  |  |
|  | Furnace for steels hardening in integrated thermal bath. Helium gas atmosphere is used. |  |  |  |  |
| 1.2. | Technical specification |  |  |  |  |
| 1.2.1. | Type | Bell furnace in integrated thermal bath (item no 3). |  |  |  |
| 1.2.2. | Maximum outer size (dimensions) | L 2 m x W 2 m x H 3,5 m |  |  |  |
| 1.2.3. | Dimensions of retort | L 300 mm x W 500 mm x H 300 mm |  |  |  |
| 1.2.4. | Temperature | Max 1050 oC |  |  |  |
| 1.2.5. | Working temperature | Range from 600-1000 oC |  |  |  |
| 1.2.6. | Bath temperature | Max 300 oC |  |  |  |
| 1.2.7. | Gas atmosphere | Technical helium (He), tank up to 40 l, pressure up to 200 bar |  |  |  |
| 1.2.8. | Connections | Power 50 kWm 230-400 V |  |  |  |
| 1.2.9. | Process steering | Programme-controller, PLC, automatic |  |  |  |
| 1.2.10. | Certificate | Applied EN 746 |  |  |  |
| 2 | FURNACE WITH ARGON GAS ATMOSPHERE, Quantity 1 |  |  |  |  |
| 2.1. | General specification |  |  |  |  |
|  | Furnace for steels hardening in integrated thermal bath. Argon gas atmosphere is used. |  |  |  |  |
| 2.2. | Technical specification |  |  |  |  |
| 2.2.1. | Type | Pit-type furnace with argon gas atmosphere for hardening |  |  |  |
| 2.2.2. | Dimensions of retort | Φ 600 x 800 mm |  |  |  |
| 2.2.3. | Outer size (dimensions) | L 1,5m x W 1,5m x H 1,5 m |  |  |  |
| 2.2.4. | Temperature | Max 1050 oC |  |  |  |
| 2.2.5. | Working temperature | Range from 200-1000 oC |  |  |  |
| 2.2.6. | Gas atmosphere | Technical argon (Ar), tanks up to 40 l, pressure up to 200 bar |  |  |  |
| 2.2.7. | Connections | Electricity 30 kW, 230-400 V |  |  |  |
| 2.2.8. | Process steering | programme-controller, PLC, automatic |  |  |  |
| 2.2.9. | Certificate | Applied EN 746 |  |  |  |
| 3 | THERMAL BATH FOR QUENCHING AND TEMPERING, Quantity 1 |  |  |  |  |
| 3.1. | General specification |  |  |  |  |
|  | Thermal bath is used for quenching and tempering of steels using oil or melted salt. |  |  |  |  |
| 3.2. | Technical specification |  |  |  |  |
| 3.2.1. | Type | Open thermal bath |  |  |  |
| 3.2.2. | Dimensions of bath | Φ 1 m x 1,2 m |  |  |  |
| 3.2.3. | Outer size (dimensions) | Φ 1,3m x 1,5 m |  |  |  |
| 3.2.4. | Temperature | Max 300 oC -heating inside with electrical heaters -cool outside double skin side walls for circulating water |  |  |  |
| 3.2.5. | Max Bach | 200 kg including weight of charging devices that are processed - item 5, item 6 |  |  |  |
| 3.2.6. | Connections | Power 25 kW, 230-400 V |  |  |  |
| 3.2.7. | Process steering | Electronically regulator |  |  |  |
| 3.2.8. | Certificate | Applied EN 746 |  |  |  |
| 4 | QUENCHING BATH – WATER, Quantity 1 |  |  |  |  |
| 4.1. | General description |  |  |  |  |
|  | Open basin for steels hardening in water. |  |  |  |  |
| 4.2. | Technical specification |  |  |  |  |
| 4.2.1. | Type | Open basin |  |  |  |
| 4.2.2. | Dimensions of bath (useful zone) | L 1m x W 1m x H 1,2 m |  |  |  |
| 4.2.3. | Outer size (dimensions) | L 1,5 m x W 1, 5m x H 1,5 m |  |  |  |
| 4.2.4. | Temperature | Max 60 oC for water heating |  |  |  |
| 5 | CHARGING DEVICE 1, Quantity 5 |  |  |  |  |
| 5.1. | General description |  |  |  |  |
|  | Welded metal constructions adjusted to the furnace entrance used for manipulation of batches that are processed in furnaces (item 1) and baths (item 3, item 4). Charging devices are consumables and the validity period is dependent on way of their use. |  |  |  |  |
| 5.2. | Technical specification |  |  |  |  |
| 5.2.1. | Type | Welded charge supports |  |  |  |
| 5.2.2. | Material | Heat resistant steel WNr. 1.4841 |  |  |  |
| 5.2.3. | Dimensions | L 300 mm x W 500 mm x H 300 mm |  |  |  |
| 6 | CHARGING DEVICE 2, Quantity 3 |  |  |  |  |
| 6.1. | General description |  |  |  |  |
|  | Welded metal constructions adjusted to the furnace entrance used for manipulation of batches that are processed in furnaces (item 2) and baths (item 3, item 4). |  |  |  |  |
| 6.2. | Technical specification |  |  |  |  |
| 6.2.1. | Type | Welded charge supports |  |  |  |
| 6.2.2. | Material | Heat resistant steel WNr. 1.4841 |  |  |  |
| 6.2.3. | Dimensions | Φ 600 mm x 800 mm |  |  |  |
| 7 | CHARGE TRANSFER EQUIPMENT, Quantity 1 |  |  |  |  |
| 7.1. | General description |  |  |  |  |
|  | Device for charge transport to furnace and bath. Self-standing device with metal construction to which the charges are connected and transferred within the hardening process. |  |  |  |  |
| 7.2. | Technical specification |  |  |  |  |
| 7.2.1. | Type | Elevating device with rotating possibility |  |  |  |
| 7.2.2. | Charge weight | Max 200 kg |  |  |  |
| 7.2.3. | Transfer options: |  |  |  |  |
| 7.2.3.1. | Horizontal | 10 m left-right directions |  |  |  |
| 7.2.3.2. | Rotation | Up to 180° |  |  |  |
| 7.2.4. | Lifting height | Up to 3 m |  |  |  |
| 7.2.5. | Transfer type | Pneumatic and electro-mechanical |  |  |  |
| 7.2.6. | Connections |  |  |  |  |
| 7.2.6.1. | Power supply | Up to 1 kW |  |  |  |
| 7.2.6.2. | Pressure supply | Up to 6 bar |  |  |  |
| All items | INSTALLATION AND TRAINING ON BASIC OPERATION OF MACHINE |  |  |  |  |
|  | All equipment must be installed by supplier at the beneficiary’s premises. The supplier must ensure training on basic operation of machine to the beneficiary. |  |  |  |  |
| All items | MANUALS |  |  |  |  |
|  | Manuals in Croatian |  |  |  |  |
| All items | WARRANTY |  |  |  |  |
|  | One year after provisional acceptance in accordance with the conditions laid down in Article 32 of the General Conditions |  |  |  |  |

Lot 2 – Table Appliance for Hardness Measuring

| 1. Item Number | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | TABLE APPLICANCE FOR HARDNESS MEASURING, Quantity 1 |  |  |  |  |
| 1.1. | General specifications |  |  |  |  |
|  | Appliance for measuring metal material hardness. Part of device is extruded to the sample surface measuring depth of the mark. The appliance is used in metal processing facilities on the thermo processed steels. High level of test result reproducibility. |  |  |  |  |
| 1.2. | Technical specification |  |  |  |  |
| 1.2.1. | Measuring methods applied | Rockwell – HRC, HBD methods |  |  |  |
| 1.2.2. | Sample dimensions | Maximum height 250 mm |  |  |  |
| 1.2.3. | Sample tightening | manually |  |  |  |
| 1.2.4. | Digital display included; |  |  |  |  |
| 1.2.4. | Charger included 240V AC 50 Hz; |  |  |  |  |
| 1.2.4. | Measuring unit for N3 for load from range 15-187,5 kg included; |  |  |  |  |
| 1.2.5. | Testing probe for HRB method with MPA certificate included; |  |  |  |  |
| 1.2.6. | Testing probe for HRC method with MPA certificate included; |  |  |  |  |
| All items | INSTALLATION AND TRAINING ON BASIC OPERATION OF DEVICE |  |  |  |  |
|  | All equipment must be installed by supplier at the beneficiary’s premises. The supplier must ensure training on basic operation of machine to the beneficiary. |  |  |  |  |
| All items | MANUALS |  |  |  |  |
|  | Manuals in Croatian |  |  |  |  |
| All items | WARRANTY |  |  |  |  |
|  | One year after provisional acceptance in accordance with the conditions laid down in Article 32 of the General Conditions |  |  |  |  |

Lot 3 – Dry Ice Machine

| 1. Item Number | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | DRY ICE MACHINE, Quantity 1 |  |  |  |  |
| 1.1. | General specifications |  |  |  |  |
|  | Electrically and air operated single hose dry ice blasting machine. Operating air pressure of maximum 14 bar and electrical power up to 1kW. To be used for cleaning of components after heat treatment process. |  |  |  |  |
| 1.2. | Technical specification |  |  |  |  |
| 1.2.1. | Material | Welded stainless steel construction |  |  |  |
| 1.2.2. | Dimensions | Maximum height 1,5 m |  |  |  |
| 1.2.3. | Compressed air connection | Claw coupling according to DIN 3489 |  |  |  |
| 1.2.4. | Operating pressure | Maximum 14 bar |  |  |  |
| 1.2.5. | Dry ice consumption | Up to 100 kg/h |  |  |  |
| 1.2.6. | Compressed air consumption | Up to 12 m3/min |  |  |  |
| 1.2.7. | Power connection | 230V, 50/60 Hz, CEE 714 Type F |  |  |  |
| 1.2.8. | Blast pressure regulation included |  |  |  |  |
| 1.2.9. | Dry Ice regulation included |  |  |  |  |
| 1.2.10. | Hose package (standard length up to 7 m) with blast gun included |  |  |  |  |
| 1.2.11. | Round nozzle included |  |  |  |  |
| 1.2.12. | Container for dry ice storage included; dimensions: 1000 mm x 1000 mm x 1500 mm |  |  |  |  |
| All items | INSTALLATION AND TRAINING ON BASIC OPERATION OF MACHINE |  |  |  |  |
|  | All equipment must be installed by supplier at the beneficiary’s premises The supplier must ensure training on basic operation of machine to the beneficiary. |  |  |  |  |
| All items | MANUALS |  |  |  |  |
|  | Manuals in Croatian |  |  |  |  |
| All items | WARRANTY |  |  |  |  |
|  | One year after provisional acceptance in accordance with the conditions laid down in Article 32 of the General Conditions |  |  |  |  |